Protocol – Transport of Fuel

INTRODUCTION

This protocol has been prepared to ensure employees who transport fuel for work purposes stay safe and within any legal requirements.

- Responsibilities

All employees have the responsibility for their own safety, the safety of other employees and those who may be effected by the services we provide. Employees and managers should ensure they do not exceed any limits or controls indicated in this protocol.

- Maximum Quantities

Carriage of fuel for work purposes is exempt from the general restrictions on carrying goods by road under the Carriage of Dangerous Goods and Use of Transportable Pressure Equipment Regulations 2009, provided that the following is complied with:

- The total quantity of fuel transported must not exceed 333 litres (petrol) or 1,000 litres (diesel);

- The fuel must be transported in individual containers that comply with the relevant United Nations Economic Commission for Europe (“UNECE”) specifications and are marked with the UN number for petrol (UN1203) or diesel (UN1202), as appropriate;

- The containers must be stored in the vehicle in a manner which avoids damage or loss from the vehicle;

- Containers

Suitable portable containers:

- are marked or labelled in a legible and indelible form with the words “Petrol” or Diesel and “highly flammable”

- have an appropriate hazard warning sign,

- have the nominal capacity in litres;

- have the manufacturer’s name; and date and month of manufacture.

The capacity of suitable portable containers must be no greater than:

• 10 litres, if made of plastic; or

• 20 litres, if made of metal, although the total capacity may exceed the nominal capacity by between 10% and 15%.

- Training

If a vehicle is carrying loads in excess of 330 litres of petrol or 1000 litres of diesel the driver must undergo specialised training.

- Fire

The vehicle must carry a multi-purpose powder extinguisher of no less than 2kg and the driver should be trained in its use.

Drivers should only tackle a fire if it is safe to do so otherwise move a safe distance from the vehicle and ensure no-one else approaches the vehicle.

No-one in the vehicle should have a naked flame or other device that could potentially cause ignition such as a lighter, matches or e-cigarette.

- PPE

Drivers must use the PPE stated in the associated COSHH assessment.

- Spillage

The vehicle shall have a spillage kit on board at all times when fuel is being transported.

Record of Change:

| Version | Authorised By | Date | Distributed To | Distribution Date |
| --- | --- | --- | --- | --- |
| 1 | People & Finance Manager | Oct 17 | All employees | Nov 17 |