| Title: GC6 Risk Assessment for Working at Height Info Sheet – Version 2 This assessment details control measures in place for work organised and carried out at height. Multiple activities including maintenance and cleaning are carried out at height. Contractors may also come on site to access equipment for maintenance or repairs. Working at height, in terms of severity, could lead to major life changing injuries or fatalities. Robust and specific control measures will be in place for specific work to ensure the safety of those undertaking the task and those who could be impacted by it. Risk assessments are also drafted for specific work undertaken in the Centre where working at height is involved. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hazard | People at risk | Control Measures in place | Additional Controls | Severity | Probability | Risk Rating |
| Falls from height | Staff Contractors Visitors Customers | - Sufficient guardrails in place at all stairs - All handrails checked regularly - Working from height avoided where possible - Appropriate system for working at height in place including specific risk assessment for the tasks carried out where applicable |  | 5 | 2 | M |
| Environmental Conditions Working on the roof – weather conditions Noisy conditions Presence of toxic gases R– 123A, FUMES Heat stroke/hypothermia | Staff Contractors | - No work on the roof to be carried out during adverse weather conditions - Personnel undertaking work to wear appropriate PPE - Adequate ventilation to be ensured in work space - Personnel to wear appropriate clothing - Personnel to ensure they take adequate breaks for hydrating |  | 5 | 2 | M |
| Falling Objects Tooling and debris falling Kit not secured correctly | Staff Customers Visitors Public | - Contractors to be trained and qualified to undertake work - Personnel to ensure all tooling is secured - Exclusion zone to be setup around work area - All kit secured to be checked to ensure it is secured correctly - Regular checks where possible of equipment secured at height - Areas including plant room and roof classified as restricted spaces |  | 5 | 2 | M |
| Fragile surfaces Pool area roof | Staff Contractors | - No access to roof above pool area (signage in place) - Staff given training prior to commencing any work - No access to roof above reception (signage and barriers in place) - Areas including roof to be accessed by scaffold where practical - No access to internal reception roof (signage and barriers in place) |  | 5 | 1 | L |
| Faulty equipment Damaged ladders Faulty work platforms | Staff Contractors | - All ladders to be inspected quarterly - All work at height platforms and lifts to be inspected and serviced by trained and competent service engineers - Ladders to be inspected by person using it prior to use - All faulty equipment to be put out of use and disposed or made inoperable by maintenance engineers |  | 4 | 1 | L |
| Inexperienced staff Untrained staff Unauthorised access | Staff Contractors | - Staff to be given training prior to working at height - All work off ladders to be done in 30 minute sessions with a 5 minute break where other work can be carried out - Staff with mobility issues to be restricted from working at height - Contractors to provide – approved IPAF certification for their engineers prior to using Elevated platforms - Mobile Work platforms and lifts to be operated by fully qualified staff only with IPAF certification |  | 4 | 1 | L |
| Collision with other physical features or personnel Crushing injury Collision with pillars and other building features Uneven surfaces | Staff Contractors Visitors Customers Public | - Banksman or responsible person to be used, where practical, to direct larger MEWP’s or Vertical Platforms safely into position - Work to be carried out during quiet periods or out of hours - Personnel undertaking the work carry out specific risk assessment and identify hazards - Complicated tasks to have a method statement and route map - Visual check of all load bearing surfaces |  | 5 | 1 | L |
| Suspension Syncope/Trauma | Contractors Staff | - Any work at height involving safety harnesses and suspension kit to be carried out by trained personnel - No centre staff to undertake work involving use suspension harnesses - Contractors to provide detailed rescue plan including medical intervention and be trained to recognise Suspension Syncope |  | 4 | 1 | L |

| ASSESSOR(s) PRINT NAME/JOB TITLE: | S MATHEW General Manager | ASSESSOR(S) SIGNATURE: | S Mathew | ASSESSMENT DATE: | 11/10/2021 |
| --- | --- | --- | --- | --- | --- |
| MANAGER PRINT NAME/JOB TITLE: | D HASSON Dep General Manager/ C COLLINS/ OPERATIONS MANAGER | MANAGERS SIGNATURE: | D Hasson/ C Collins | REVIEW DATE: | 17/04/2024 |