| Title: AZ4 Use of electrical equipment in wetside areas (incl water cooler) Version 4.0 Electrical equipment such as cleaning equipment, water coolers, fans and blowers, portable sound systems are used in the swimming pool area. The overall aim is to minimise the use of electrical equipment in wet side areas as much as possible. and the use of waterproof and immersion proof equipment poolside areas is best practice. Reviewed and updated by C Collins, D Hasson and S Mathew on 28 May 2024 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hazard | People at risk | Control Measures in place | Additional Controls | Severity | Probability | Risk Rating |
| Electricity Damaged equipment Exposed Cabling Personal electrical equipment Contact with live equipment | Swimmers Staff | - The main Socket outlets should not normally be located in wet areas. However, where this is necessary they must be designed for this type of environment. - Any electrical equipment used near the poolside should be designed to withstand immersion. If this is not possible, it must only be used when the pool is not occupied and equipment protected from falling into the pool e.g. chain or a barrier in place. - Appliances including audio equipment should be located in areas on the poolside where contact with water can be minimised - Isolating facilities should be provided to enable parts of the installation to be disconnected to allow for routine maintenance/repair. - It is important that the electrical installation and portable appliances are maintained in a safe condition. - All electrical items used within the Centre to be PAT tested (exception for personal equipment of customers) - All equipment checked prior to use in the pool area for any physical damage. Any damaged equipment to be removed, repaired or replaced - Circuit breakers used on all equipment - All Poolside sockets are Residual Current Devices (RCD) - Poolside subject to annual Fixed Wire Testing by an external contractor - All electrical works carried using safe working practices as detailed in Maintenance and Cleaning risk assessments and should comply with Electricity at Work Regulations 1989- https://www.hse.gov.uk/pubns/priced/hsr25.pdf |  | 5 | 2 | M |
| Slips trips and falls Cables causing trip hazards Equipment not stored correctly | Visitors Swimmers Staff | - All trailing cables to be highlighted by use of signage and warning posts, where possible cable covers should be used. - When cleaning using scrubbers, area to be closed off. Trailing cables to be confined to within barriered area. - Cordless Equipment preferable for daily use within the Centre - Appropriate Storage for all equipment away from the pool - Daily inspection of the poolside surroundings to ensure poolside and high traffic areas are clear of any tripping hazards - All equipment for maintenance purposes kept under supervision of Maintenance supervisor. Cleaning equipment such as Pressure Washers can be stored safely in Poolside Staffroom and plant room. |  | 3 | 2 | L |
| Water cooler Spills and Hygiene issues Misuse by customers Customers from spectator area coming to poolside to access water cooler | Swimmers Staff | - Cooler to be fitted by qualified personnel in line with manufacturers guidance with and connected via an RCD - Power supply point to be boxed in to avoid any water spills or inadvertent contact by staff or swimmers - Area around water cooler to be kept clean and checked by centre attendants on rounds. Any build up during intervals to be highlighted and addressed by a Dryside attendant - Equipment to be tested as per regulations and replaced in case of any damage - Any spills to be notified to cleaning staff and area cordoned off till spill has been cleared - Use of correct PPE during checks and repairs if any maintenance work is required - Water Cooler Subject to quarterly inspections from Waterpoint (supplier) - Visiting Contractor should be issued with appropriate Health and Safety Documentation. (Contractor induction pack) |  | 3 | 2 | L |

| ASSESSOR(s) PRINT NAME/JOB TITLE: | C COLLINS/ OPERATIONS MANAGER | ASSESSOR(S) SIGNATURE: | C COLLINS | ASSESSMENT DATE: | 23/07/2020 |
| --- | --- | --- | --- | --- | --- |
| MANAGER PRINT NAME/JOB TITLE: | D HASSON Dep General Manager/ S MATHEW General Manager | MANAGERS SIGNATURE: | D HASSON / S MATHEW | REVIEW DATE: | 28/05/2024 |